| S.No. | Experiment Name / Major Topics / Minor Topics Included | Output to be Checked | TYPE OF EXPT |
| --- | --- | --- | --- |
| 4 | Simulation and Comparative Performance Evaluation of Zigbee, 6LoWPAN and NB-IoT Communication Protocols for IoE Applications using NS-3 Major: LPWAN and Low-Power Networks Minor: Zigbee, 6LoWPAN, NB-IoT, packet delivery, delay | NS-3 simulation results; throughput, delay, packet-delivery ratio and energy comparison; performance graphs | SIMULATION |

import numpy as np

import pandas as pd

import matplotlib.pyplot as plt

from IPython.display import display

import os

import zipfile

np.random.seed(42)

# Experiment settings

NUM_NODES = 50

SIM_TIME = 100

PACKET_BYTES = 100

PACKET_BITS = PACKET_BYTES * 8

TOTAL_PACKETS = NUM_NODES * SIM_TIME

# Illustrative protocol parameters (educational model)

protocols = {

"Zigbee":  {"pdr": 0.94, "delay": 35,  "jitter": 12,

"rate": 250000, "tx": 17.4, "rx": 18.8,

"idle": 0.02, "voltage": 3.0, "overhead": 1.0},

"6LoWPAN": {"pdr": 0.96, "delay": 45,  "jitter": 15,

"rate": 250000, "tx": 16.0, "rx": 17.0,

"idle": 0.02, "voltage": 3.0, "overhead": 1.15},

"NB-IoT":  {"pdr": 0.985, "delay": 180, "jitter": 60,

"rate": 60000, "tx": 220.0, "rx": 40.0,

"idle": 0.005, "voltage": 3.6, "overhead": 1.5}

}

os.makedirs("IoE_Experiment_Results", exist_ok=True)

outdir = "IoE_Experiment_Results"

rows = []

for name, p in protocols.items():

received = int((np.random.rand(TOTAL_PACKETS) < p["pdr"]).sum())

delays = np.clip(

np.random.normal(p["delay"], p["jitter"], received), 1, None

)

avg_delay = np.mean(delays)

pdr_percent = received / TOTAL_PACKETS * 100

throughput = received * PACKET_BITS / SIM_TIME / 1000

# Approximate energy model

tx_time = PACKET_BITS / p["rate"] * p["overhead"]

rx_time = PACKET_BITS / p["rate"]

tx_energy = SIM_TIME * p["voltage"] * p["tx"] / 1000 * tx_time

rx_energy = SIM_TIME * p["pdr"] * p["voltage"] * p["rx"] / 1000 * rx_time

idle_energy = (

p["voltage"] * p["idle"] / 1000

* max(0, SIM_TIME - SIM_TIME * tx_time)

)

energy_node = tx_energy + rx_energy + idle_energy

rows.append({

"Protocol": name,

"Packets Sent": TOTAL_PACKETS,

"Packets Received": received,

"PDR (%)": pdr_percent,

"Throughput (kbps)": throughput,

"Average Delay (ms)": avg_delay,

"Estimated Energy per Node (J)": energy_node,

"Estimated Network Energy (J)": energy_node * NUM_NODES

})

df = pd.DataFrame(rows)

print("IoE PROTOCOL COMPARATIVE PERFORMANCE")

print("Python educational simulation — not actual NS-3 results")

print(f"Nodes: {NUM_NODES}, Duration: {SIM_TIME} seconds, "

f"Packet size: {PACKET_BYTES} bytes\n")

display(df.round(3))

print("\nSUMMARY")

print("Highest PDR:", df.loc[df["PDR (%)"].idxmax(), "Protocol"])

print("Highest throughput:", df.loc[df["Throughput (kbps)"].idxmax(), "Protocol"])

print("Lowest delay:", df.loc[df["Average Delay (ms)"].idxmin(), "Protocol"])

print("Lowest estimated network energy:",

df.loc[df["Estimated Network Energy (J)"].idxmin(), "Protocol"])

# Save results table

df.to_csv(f"{outdir}/comparison_results.csv", index=False)

# Generate and save all four performance graphs

graphs = [

("Throughput (kbps)", "Throughput Comparison", "Throughput (kbps)"),

("Average Delay (ms)", "End-to-End Delay Comparison", "Delay (ms)"),

("PDR (%)", "Packet Delivery Ratio Comparison", "PDR (%)"),

("Estimated Network Energy (J)", "Estimated Energy Comparison", "Energy (J)")

]

for column, title, ylabel in graphs:

plt.figure(figsize=(8, 5))

plt.bar(df["Protocol"], df[column])

plt.title(title)

plt.xlabel("Protocol")

plt.ylabel(ylabel)

plt.grid(axis="y", alpha=0.3)

if column == "PDR (%)":

plt.ylim(0, 105)

plt.tight_layout()

filename = column.lower().replace(" ", "_").replace("(", "").replace(")", "")

plt.savefig(f"{outdir}/{filename}.png", dpi=200, bbox_inches="tight")

plt.show()

plt.close()

# Put the CSV and graph images into one ZIP file

with zipfile.ZipFile("IoE_Experiment_Results.zip", "w") as z:

for filename in os.listdir(outdir):

z.write(os.path.join(outdir, filename), arcname=filename)

print("\nFiles created successfully:")

print("1. IoE_Experiment_Results/comparison_results.csv")

print("2. Four graph PNG files in IoE_Experiment_Results/")

print("3. IoE_Experiment_Results.zip")